- Consider a Line of sight SIMO AWGN channel with four element receive array. The elements of the channel vector h are stated in the table below and satisfy the conditions |hi|2 = 1. i = 1, 2, ... , NR. The power input into the transmitter is 2 watts and the noise power can be assumed as 0.004 watts per link.

|  | h11 | h21 | h31 | h41 |
| --- | --- | --- | --- | --- |
| Ampl(Norm) | 0.877 | 0.5 | 0.4 | 0.6 |
| Delay nsecs | 150 | 160 | 170 | 180 |

- Determine the capacity of this SIMO channel when h is known both at the receiver and transmitter.

- If instead this was a MISO link would the capacity increase or decrease? Why?

- Consider a Line of sight MISO AWGN channel with four element transmit array. The elements of the channel vector h are stated in the table below and satisfy the conditions |hi|2 = 1. i = 1, 2, ... , NT. The power input into the transmitter is 2 watts and the noise power can be assumed as 0.004 watts per link.

|  | h11 | h12 | h13 | h14 |
| --- | --- | --- | --- | --- |
| Ampl(Norm) | 0.48 | 0.5 | 0.4 | 0.6 |
| Delay nsecs | 150 | 160 | 170 | 180 |

- Determine the capacity of this MISO channel when h is known both at the receiver and transmitter. If instead this was a SIMO link would the capacity increase or decrease? Why?

- Determine the capacity of the MISO system if h was known only at the receiver.

3. The receive values of a MIMO system is shown under a Frequency non-selective, slow fading channel conditions. Determine the number paths and the condition number.

| Rx Tx | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 1 | 1.04 | 0.5 | 0 |
| 2 | 0.5 | 1.04 | 0.5 |
| 3 | 0 | 0.5 | 1.04 |

If the total transmit power is limited to 2 watt determine the power distribution and SNR of each path ( No = 0.04 watts/Hz) and the capacity.

3.